# KiCad 9 Beginner Tutorial for Nano Ardule / FluidCanvas

# Part 1. Create an empty project

1. Open KiCad 9 and start the Project Manager.

2. Click File → New Project.

3. Choose a folder and name (example: NanoArdule_FluidCanvas).

4. Open the Schematic Editor from the project window.

# Part 2. Prepare the schematic sheet

1. File → Page Settings.

2. Fill Title, Comments, or project description if desired.

3. Save the schematic (Ctrl+S).

# Part 3. Learn the essential tools

Important tools in Schematic Editor:

A – Add Symbol

P – Add Power Symbol

W – Draw Wire

L – Place Net Label

J – Place Junction

These few tools are enough for most schematic work.

# Part 4. Place main blocks first

Place major blocks before wiring:

• Arduino Nano (center)

• MIDI IN

• MIDI OUT

• microSD module

• I2C LCD

• Rotary encoder

• Six switches

• LEDs

# Part 5. Add the Arduino Nano

Press A (Add Symbol).

Search for Arduino Nano symbol.

If unavailable, temporarily use two generic headers (Conn_01x15).

Label pins manually if necessary.

# Part 6. Add power symbols

Press P → select +5V and GND.

Place these symbols near power connections.

Power symbols automatically connect nets of the same name.

# Part 7. Add MIDI connectors

Add two 5‑pin connectors.

Rename them J_MIDI_IN and J_MIDI_OUT.

# Part 8. Add MIDI input circuitry

Place symbol for 6N138 optocoupler.

Place symbol for 74HC14 Schmitt trigger buffer.

Arrange them between MIDI IN connector and Arduino.

# Part 9. Add microSD connection

Use a generic connector such as Conn_01x06.

Rename it J_uSD_MODULE.

Typical signals: MOSI, MISO, SCK, CS_SD, +5V, GND.

# Part 10. Add I2C LCD connector

Place Conn_01x04 and rename to J_LCD_I2C.

Signals: GND, +5V, SDA, SCL.

# Part 11. Add rotary encoder

Search for Rotary_Encoder symbol.

Place it near the control section.

Pins: A, B, Common (and optional push switch).

# Part 12. Add six push buttons

Use SW_Push symbols.

Place six switches and rename SW1–SW6.

# Part 13. Add LEDs

Place LED symbols.

Add a resistor in series with each LED.

# Part 14. Add resistors and capacitors

Place resistors for LEDs and MIDI circuits.

Place 100 nF capacitors for power decoupling.

# Part 15. Use Net Labels

Press L to place Net Labels.

Typical names: MOSI, MISO, SCK, CS_SD, SDA, SCL, LED1, SW1, ENC_A.

# Part 16. Wire the schematic

Connect power first.

Connect MIDI IN chain: DIN → 6N138 → 74HC14 → Arduino.

Connect MIDI OUT: Arduino TX → DIN.

Connect SPI lines for SD module.

Connect switches and LEDs to Arduino pins.

# Part 17. Edit values and references

Double‑click components to edit values.

Examples: 220R, 10k, 100nF.

# Part 18. Annotate components

Tools → Annotate Schematic.

KiCad assigns R1, C1, U1 etc.

# Part 19. Run Electrical Rules Check

Inspect → Electrical Rules Checker.

Fix warnings such as unconnected pins.

# Part 20. Final layout idea

Arrange schematic blocks logically:

Left: MIDI connectors

Center: Arduino Nano

Top right: LCD and SD

Bottom right: switches, encoder, LEDs

The goal is a clear and readable schematic.